Scope

To reduce and eliminate where possible the risk of incorrect packaging and / or labelling of finished product reaching the consumer.

Responsibilities

| Employee | Responsibilities |
| --- | --- |
| QA Manager / Technical Services Manager | Ensured this procedure covers customer requirements and that the QA team are completing all required checks necessary |
| Slicing Managers | Ensured the procedure is followed by all designated operatives |
| Line Leaders | Ensured that the machine is switched on and the current product is loaded into the machine at the start of each production run |
| QA Operatives | Responsible to conduct all fail-safe checks on all machines as per required frequency and that all products which cannot go through the vision systems are checked on the offline vision system |

Frequency of Testing

All Fail Safe checks are to be conducted at the start of production, end of production, and after any repairs, maintenance conducted on the line (Incorrect barcode and 2D matrix at the start of the shift). If a line starts production with a product that does not run through the online vision system or ends production with one, the QA’s must be sure to complete the fail-safe checks when retail packs start or end.

All products that cannot be placed down the online machine e.g., 1kg packs, gammon joints not packed in a box, must be checked at the start of the product run, every 30 mins and at the end of the product run on the offline Vision System machine by a QA.

Vision System: crate end labels – Outer Case Code OCC’s

A crate end labels check must be completed by trained production operative, by scanning a crate end label on Vision system at the same time when a label check is completed on the packing line. This is to confirm that they are correct for the product running or to be run on the packing line. The frequency of OCC label checks is listed below:

- At the start of product run during the first products on the line, otherwise the line will stop

- Every 30 minutes

- Each time the outer case label information is changed

- At the end of the production run

In case of the wrong label found when scanning line will stop. This must be escalated to QA operatives working on the shift. All products since the last successful check must be help and quarantined following the FSP33 Non-conformance procedure and investigated.

All labels for control on Vision System must be collected from the point where they are used. Under no circumstances the labels can be taken from the place on the table, for the use of recording checks.

Procedure

Checks must be completed during the production run for each product:

•	All steps must be enabled as per customer requirements

•	None of the machines are in override (unless producing 1kg packs)

•	All products have been trained in to highlight the difference in each product e.g., Unsmoked Wiltshire Back is set up on product name to U, W, B

Fail Safe Checks

1-Verification of line stop system for consecutive fails: (consecutive miss-reads/serial alarm)

•	Ensure that the line is running normally.

•	Rotate consecutive packs 180° prior to the unit until the belt stops and the line stop alarm is triggered. This should take place after 5 consecutive rejections, alarms  ‘Serial Alarm’

•    If the belt doesn’t stop inform the QA Supervisor / QA Manager & Engineers so the appropriate corrective action can be taken

2-Verification of reject failure: Reject confirmation (Air Pressure Flow)

•	Ensure that the line is running normally.

- Disconnect the air from the reject arm.

•	If the belt stops then the check is successful.

•	If the belt doesn’t stop inform the QA Supervisor / QA Manager & Engineers so the appropriate corrective action can be taken

3-Verification of incorrect base barcode

•	Ensure that the line is running normally.

•	Load a product into the system

•	Send through a dummy pack which has a different barcode on (this can be a pack from another line that is running or if not, a dummy pack that has been made up must be used)

•	If the belt stops the check is successful.

- If the belt doesn’t stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

4- Verification of incorrect 2D Matrix

•	Ensure that the line is running normally.

•	Load a product into the system, ensure that the data matrix step is active

•	Send through a dummy pack which has a different 2D barcode on (this can be a pack from another line that is running or if not, a dummy pack that has been made up must be used)

•	If the belt stops then the check was successful, and alarms ‘Wrong Barcode’

•	If the belt doesn’t stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

5-Bin Full Sensor

•    Ensure that the line is running normally

•	Cover the bin full sensor to see if belt stops.

•	If the belt stops then the check is successful.

•	If the belt doesn’t stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

6-Verification of reject bin confirmation

•	Ensure the line is running normally

•	Place a pack into the Vision system machine so the camera identifies the pack

•	The pack must be taken away, so it does not reject into the bin

•	If the arm goes to reject the pack and no pack enters the bin, then the belt should stop, and alarms’ Reject confirmation’ triggers

•	If the belt stops then the check was successful

•	If the belt doesn’t stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

7-Verification of infeed sensor (The infeed sensor is located at the entry point of the machine)

•	Ensure the line is running normally

•	Cover the sensor to see if the belt stops and alarms ‘Pack sensor Covered’

•	If the belt stops then the check was successful.

•	If the belt does not stop inform the QA Supervisor / QA Manager & an Engineer so that the appropriate corrective action can be taken

8-Packaging Check

•	Top Web/Sleeve Check – Cover a location on the product that is trained into the label verifier.

•	Base Label/Sleeve Check – Send a pack down the line covering the barcode on the base label/Sleeve.

•	Promo/Flash Label Check – Send a pack down the line covering any Promo/Flash label on the top web/sleeve (if applicable)

•	Incorrect Date Code – Send a pack down the line with one digit of the date code.

•    All packs should be rejected when the tests are carried out. If any of the failsafe checks are unsuccessful, then the QA

Supervisor / QA Manager must be informed immediately so that the appropriate corrective action can be taken.

9-Reject Confirmation Sensor Check

•   Ensure the line is running normally

•   Cover the sensor with the handle to see if the belt stops

•    If the belt stops then the check was successful.

•    If the belt does not stop inform the QA Supervisor / QA Manager & an Engineer so that the appropriate corrective action can

be taken

Fail Safe Checks Line 9 Spark Eye

- Incorrect Date Code – Send a pack down the line with one digit of the date code. The pack should be rejected

2 - Verification of reject bin confirmation

•	Ensure the line is running normally

•	Place a pack into the Spark Eye system so the camera identifies the pack

•	The pack must be taken away, so it does not reject into the bin

•	If the arm goes to reject the pack and no pack enters the bin, then the belt should stop

•	If the belt stops then the check was successful, and alarms’ Reject confirmation’ triggers.

•	If the belt does not stop, inform the QA Supervisor, QA Manager, and an Engineer so that appropriate corrective action can be taken

3-Bin Full Sensor

•    Ensure that the line is running normally

•	Pull the bin out slightly and this will cover the sensor and stop and alarm ‘Bin full’ triggers.

•	If the belt stops then the check is successful.

•	If the belt does not stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

4-Verification of infeed sensor (The infeed sensor is located at the entry point of the machine)

•	Ensure the line is running normally

•	Cover the sensor to see if the belt stops and alarm ‘Photo eye block’ triggers.

•	If the belt stops then the check was successful.

•	If the belt does not stop inform the QA Supervisor / QA Manager & an Engineer so that the appropriate corrective action can be taken

5- Verification of incorrect 2D Matrix

•	Ensure that the line is running normally.

•	Load a product into the system, ensure that the data matrix step is active

•	Send through a dummy pack which has a different 2D barcode on (this can be a pack from another line that is running or if not, a dummy pack that has been made up must be used)

•	If the belt stops, the check is successful, and the alarm ‘Incorrect Code’ triggers.

•	If the belt does not stop inform the QA Supervisor / QA Manager & an Engineer so the appropriate corrective action can be taken

6-Verification of line stop system for consecutive fails: (Consecutive miss-reads/Serial Alarm)

•	Ensure that the line is running normally.

•	Rotate consecutive 5 packs 180° prior to the Spark Eye until the belt stops and alarm ‘Incorrect Code’ triggers. This should take place after 5 consecutive rejections.

•    If the belt does not stop inform the QA Supervisor / QA Manager & Engineers so the appropriate corrective action can be taken

Belt Stop Risk Check

When carrying out the fail-safe checks, when the belt stops the pack must stop on the belt for the failsafe to be successful.

If the pack continues to the e.g. lazy Susan, then the failsafe check has failed.

The QA Supervisor and QA Manager must be informed immediately so that the corrective action can be taken.

If any of the failsafe checks are unsuccessful, then the QA Supervisor / QA Manager must be informed immediately so that the appropriate corrective action can be taken.

All results from the fail-safe checks must be recorded on to FSP74b - The online packaging & labelling Verification check sheet and Line 9 Spark Eye Only check sheet on Quor

Vision System/ Spark Eye Champion Responsibilities

•	To help with any issues that occur on the label verifying machines that can’t be corrected by the Line QA.

•	To liaise with Vision System when any issues occur that they cannot resolve themselves.

•	If any of the machines can’t run for any reason or a step needs to be temporally disabled, then a concession must be requested and approved as per FSP41 Concession Procedure.

•	For M&S products only – The Technical Manager must inform the M&S Technologist in the event a concession is required if any of the machines can’t run for any reason, or a step needs to be temporally disabled

Line Leader / Appointed Paperwork Person Responsibilities

•	To ensure that we are fully compliant with our Customers Codes of Practices, and to ensure that our Customers Food Safety is not compromised, all packs that are produced on the lines must be ran through the online label verifiers to be inspected and under no circumstances MUST they be took out of inspection mode by anyone other than the authorised QA.

•	Therefore, if any issues occur on the line with the Vision System/ Spark Eye, then the trained appointed QA must be informed immediately so that they can come to the machine to try to resolve the issue.

•	If the machine needs to be taken out of inspection mode by the QA for any length of time or reason, then the line leader must be informed, and they MUST start to pack all products in to baskets that have full traceability on until the issue is resolved.

•	Once the issue has been fixed and the machine can be put back into inspection mode then all products that have been packed into the baskets MUST be placed through the Vision System/Spark Eye machine to be inspected before being packed.

•	This is a customer requirement, and the last fail-safe point for label checks and date coding. It is so important that no packs are packed into final SRP (Supplier Retail Packaging) without going through a Vision System/ Spark Eye.

Corrective Action - Rejected Product

•	All rejected products must be removed from the locked bin and visually checked if any issues are found then they must be placed into the relevant pre-labelled with full traceability on basket to be re-worked.

•	If no obvious issues can be seen the pack must be placed on to the line to go through the Vision System/ Spark Eye again, if the pack is accepted it can be packed, if it is rejected again then it must be sent for re-work.

•	QA’s and the Line Leader / Paperwork person must work together to complete the downtime record with all details of what the issue was on the machine and how long the line was down for.

NB: Any products that have a derogation from the online Vision System/ Spark Eye e.g., bulk product will follow the offline Veri Central process as per procedure.

Corrective Actions - System Fails

•	If any labels are found to be incorrect it must be reported to the QA Supervisor / QA Manager and all effected products must be put on QA hold until it is all re- worked. In case of situation where issues are found by QA operatives and corrected on the line, the products do not have to be placed on hold.

•	If any of the steps / cameras or parts of the verifying system fail, then the following steps must be taken:

•	Inform the Vision System/ Spark Eye Champion, so they can try to see if the issue can be resolved.

•	If the issue cannot be resolved, then the QA Supervisor / Technical team must be informed immediately, and a concession must be raised for approval

•	Once this has been approved then the step / camera that is not working is disabled by the appropriate trained staff who have the higher level of access.

•	When the screen appears asking what the issue is then the Concession number must be entered

NB: No cameras/ steps are to be switched off or disabled before a concession is granted

•	The equipment manufacturer must then be contacted and informed of the issue so that they can carry out investigations, once the issue has been resolved the camera / step can be enabled and the concession sheet must be filed.

•	When any failure occurs with any part or all the label verifying system then manual at the start of run, every 15-minute off-line and end of run checks must be carried out by QA. These should be done throughout the production until the label verifying System has been repaired and is fully operational.

•	If any faults happen when producing any M&S product and it results in a camera or step having to be switched off by concession, then the Technical Manager must be informed of the issue so that the appropriate action can be taken following the M&S Food Manufacturing Standard – Section 6.5 Finished Product Packaging Verification and Date Code-Control

•   15-Minute Check – When the automated packaging verification system Vision system/Spark Eye are down (partially or completely) and   cannot be restored to full functionality, a trained member of the site management (shift managers or above) must carry out the label checks on paperless system Quor

Documents

•	FSP74b – Online packaging & labelling verification check sheet and Line 9 Spark Eye Only check sheet (paperless system QUOR)

•	FSP74c – Dimaco Veri-pack label Check Sheet

•	FSP41 – Concession Procedure

•    Label Book (paperless system QUOR)

•	Veri-Central Database and Audit Log

- ON-LINE LABEL CONTROL AND LABELLING COMPETENCY TEST

I can confirm I have been trained in the above procedure, and it has been translated correctly, I fully understand the contents and will adhere to the principles learnt in this training:

| Trainee Signature: | Trainee Print name: |  |
| --- | --- | --- |
| Trainer signature: | Trainer print Name: |  |
| Translator Signature: | Translator Print: | Language Translated Into: |
| Date of training: | Duration: |  |
| Reason for training: |  |  |

Review:

| Trainee Signature: | Trainee Print name: |  |
| --- | --- | --- |
| Trainer signature: | Trainer print Name: |  |
| Translator Signature: | Translator Print: | Language Translated Into: |
| Date of training: | Duration: |  |
| Reason for training: |  |  |

Amendment Log

| Date | Replaced | New Issue Number | No Of Pages | Reason | Authorised by | Training Required |
| --- | --- | --- | --- | --- | --- | --- |
| 12/01/12 | NA | 1 | 2 | New Issue | J. Bolton |  |
| 21/03/12 | V1 | 2 | 2 | Amended responsibilities following review of operation | C. Conyers | Yes, line leaders and shift managers |
| 05.05.14 | 2 | 3 | 5 | Incorporated 74a with this procedure and added in responsibilities of Lead Dimaco Champion | J. Bolton | Yes, Team Leaders, QA’s & Dimaco Champions |
| 16.05.14 | 3 | 4 | 6 | Added in Bin full sensor | J. Bolton | Yes, Team Leaders, QA’s & Dimaco Champions |
| 26.03.15 | 4 | 5 | 6 | Added in M&S technologist to be informed if any steps are disabled or the machine isn’t working | J. Bolton | yes |
| 25.06.15 | 5 | 6 | 7 | Added in corrective actions – system fails, frequency of failsafe checks added in to after repair maintenance | J. Bolton | Yes |
| 30-03-17 | 6 | 7 | 6 | Full review of procedure | M. Herriot | Yes |
| 10.07.18 | 7 | 8 | 6 | Full review of procedure | I.Radlinska | Yes |
| 03.08.19 | 8 | 9 | 6 | Included belt stop risk check | J. Bolton | Yes |
| 16.03.20 | 9 | 10 | 6 | Updated to reflect new process | J. Bolton | Yes |
| 01.12.22 | 10 | 10.1 | 7 | Updated Section 5.0 and Section 7.0 for M&S products | S. Oliver | No |
| 17.10.2024 | 10.1 | 11 | 6 | OOC labels to be controlled along with the other labels | J.Szymanski | Yes, full retraining. |
| 09/01/2024 | 11 | 12 | 6 | 30 minute off line check on automated label check changed to 15 minutes as per M&S requirement. | D. Digriene | Yes, Only QA team. |
| 30/01/2025 | 12 | 13 | 6 | Removed statement that packs rejected in to Dimaco, can be inspected and placed back to the line, Replaced with – rejected packs should be send through Dimaco again, if rejected again, to be re-worked. | D. Digriene | Yes, full training |
| 23.10.25 | 13 | 14 | 8 | Updated to reflected line 9 Spark Eye fail safe checks Reduced frequency of Vision System checks on crate end labels | B. Szimber | Yes |